# Preparing a Figure Using FIJI/ImageJ

Part 1: Image Corrections

- Open Images

  - Open file Actin-1.tif

  - Open file Actin-Flatfield.tif

- Set the Image Scale

  - Set image scale (pixel size is 0.369 μm x 0.369 μm)

  - Analyze>Set Scale

    - Distance in pixels: 1

    - Known distance: 0.369

    - Pixel aspect ratio: 1.0

    - Unit of length: micron

    - Select “Global” to apply to all open images

  - Click “OK”

- Flatfield Correction

  - Divide Actin-1.tif by Actin-Flatfield.tif to correct for uneven illumination.

  - Process>Image Calculator>Divide

    - Check off the “Create new window” option to create a new image.

    - Check off “32-bit (float) result” to ensure no intensity data is lost

  - Click “OK”

- Correct Actin-1.tif for background intensity

  - Draw a circle in a dark area of the flatfield corrected Actin-1.tif image

  - With the ROI selected go to Analyze>Measure

  - Note down the mean intensity from the results window

  - Go to Process>Math>Subtract> enter the mean intensity from the background ROI

  - Click “Apply”

  - A new image will appear.

  - Go to Image>Type>16-bit

  - Save the 16-bit image as Actin-1 -Corrected.tif

- Repeat all these steps for all of the Actin, Actin Treated, DAPI and DAPI Treated images.

Part 2: Adjust Image Display

- Adjust brightness and contrast and gamma

  - Choose one of the pairs of Actin/DAPI for the control and treated conditions.

  - For the Actin control image go to Image>Adjust>Brightness/Contrast

  - Adjust the maximum brightness

  - If possible, avoid adjusting the contrast as it is difficult to reproduce on new images

  - Since the image is background corrected the minimum should be set to zero

  - Note down the maximum brightness value

  - To adjust the gamma value go to Process>Math>Gamma

  - Try a few values to find the best one to visualize actin staining across the cells

  - Note down the gamma value used

  - Select the Actin-Treated image and adjust the brightness and gamma using the same values as the control Actin image

  - Repeat this process for the DAPI control and treated images

  - NOTE: Adjusting the gamma value will change the intensity values in the image. This operation should only be used for image display in a figure and not for quantitative image analysis.

- Add a scale bar (Pixel Size is 0.369 x 0.369 μm)

  - Go to Analyze>Tools>Scale Bar

  - Set Width in microns to 50

  - Set Height in microns to 50

  - Set thickness in pixels to 10

  - Set Font Size to 28

  - Set Color to White

  - Set Background to None

  - Set Location to Lower Right

  - Choose Horizontal, Bold Text, Overlay

  - Click “OK”

- Repeat these steps for all images. Only one scale bar is necessary if all of the images are the same size.

- Save all the images as .jpg files for the figure – Actin-1.jpg, Actin-Treated-1.jpg, DAPI-1.jpg, DAPI-Treated-1.jpg

Part 3: Create Image Colour Overlay

- Open file DAPI-1-Corrected.tif

- Open file Actin-1-Corrected.tif

- Go to Image>Color>Merge Channels – Check off “Create Composite” and “Keep Source Files”.

- Try several different colour combinations.

- Go to Image>Adjust>Colour Balance if it is necessary to adjust the relative brightness of the channels. However, if this is done, the same values must be used for the treated image overlay.

- Save the overlay as Actin-DAPI-1-Overlay.jpg

- Repeat these steps for Actin-Treated-1.tif and DAPI-Treated.tif

Part 4: Generate the Figure

- Open PowerPoint.

- Insert DAPI-1.jpg, Actin-1.jpg and the overlay.

- Insert DAPI-Treated-1.jpg, Actin-Treated-1.jpg and Actin-DAPI-1-Overlay.jpg

- Write a figure caption for the image. Metadata information for the figure caption. CHO-K1 cells, Actin stained with phalloidin AF488, FITC cube, 150 ms exposure, DAPI cube, 50 ms exposure, flat field corrected using images of a fluorescent plastic slide, upright Zeiss Axioskop, EC PlanNeoFluar 20x/0.5 NA objective lens, AxioCam ICm1 camera, brightness, gamma and contrast were adjusted to visualize the features well, the scale bar is 50 μm.

Part 5: Measure the Control and Treated Actin Intensity

- Open the DAPI-1-Corrected.tif image

- Go to Image>Adjust>Threshold

- Select “Dark background” and adjust the threshold value to capture the whole nuclei but not too much background.

- Click “Apply”

- Go to Process>Binary>Make Binary

- To fill any gaps in the DAPI intensity within the nucleus, go to Process> Binary>Fill Holes

- Go to Analyze>Analyze Particles

- Set Analyze Particles as follows:

  - Size (micron^2): 0-Infinity

  - Circularity: 0.00-1.00

  - Show: Outlines

  - Check boxes for Display Results, Clear Results, Add to Manager

- An image of the region of interest (ROI) outlines and numbers will be generated. Don’t worry if a few double nuclei show up as one ROI or is small background staining is picked up.

- Information about the ROIs will appear in the ROI Manager window

- Select the Actin-1-Corrected.tif image.

- Go to Image>Overlay>From ROI Manager

- The numbered ROIs from the DAPI-1-Corrected.tif image will appear on the Actin-1-Corrected.tif image

- Select the Results window and go to Edit>Clear. Don’t save the results from the DAPI image.

- Select the ROI Manager window. If it is not open, go to Analyze>Tools>ROI Manager

- Click “Measure”.

- In the Results window the number of each ROI and the mean intensity of actin will appear.

- There should be about 54 ROIs.

- Go to “More” button in the ROI Manger window. Click “Save” and save the ROIs as Actin-1-ROIs.tif

- In the Results window go to Edit>Copy and copy the results to an Excel spreadsheet.

- Repeat the process of measuring the actin intensity in the DAPI ROIs for all the corrected Actin images using the paired DAPI images

- Repeat the entire process on the Actin-treated dataset

Part 6: Creating a Plot of the Data using “Plots of Data”

https://huygens.science.uva.nl/PlotsOfData/

- Click the "Data Upload" tab (if not already clicked).

- Select "Paste data" on the left, after which a text input area named "Paste data below:" should appear.

- Copy-paste the columns of data from Excel (with column headers) to the "Paste data below:" text input area

- Select "Tab (from Excel)" as a delimiter

- Click "Submit Data”

- The columns of data should appear on the right and check to make sure things are in order (number of columns, rows, etc). Note that spaces in columns headers will be replaced with underscores.

- Click on the "Plot" tab, wait a few seconds (depends on connection and amount of data) and a plot should appear with all the data.

- On the left, select different ways of visualizing the data can be selected and display parameters adjusted.

- The plot can then be downloaded in an appropriate format.

- Click Data Summary to get some stats on the dataset.